Raspberry Pi 5 + Coral Edge TPU + YOLO Setup Guide

==================================================

These steps assume:

- Raspberry Pi OS is already installed and updated

- Raspberry Pi 5 (64-bit)

1. Create project folder

------------------------

mkdir -p ~/yolo

cd ~/yolo

deactivate 2>/dev/null || true

2. Install build dependencies

-----------------------------

sudo apt update

sudo apt install -y build-essential libssl-dev zlib1g-dev libbz2-dev libreadline-dev libsqlite3-dev libncursesw5-dev libgdbm-dev libffi-dev liblzma-dev tk-dev libdb-dev uuid-dev libexpat1-dev

3. Install pyenv

----------------

curl https://pyenv.run | bash

Add to ~/.bashrc:

export PATH="$HOME/.pyenv/bin:$PATH"

eval "$(pyenv init -)"

eval "$(pyenv virtualenv-init -)"

Reload shell:

exec $SHELL

4. Install Python 3.9

---------------------

pyenv install 3.9.19

cd ~/yolo

pyenv local 3.9.19

python -V

5. Create coral-env virtual environment

---------------------------------------

python -m venv coral-env

source coral-env/bin/activate

pip install --upgrade pip

6. Install Coral TPU runtime

----------------------------

Download libedgetpu1-max_16.0tf2.19.1-1.trixie_arm64.deb from:

https://github.com/feranick/libedgetpu/releases

Install:

sudo dpkg -i libedgetpu1-max_*.deb

7. Install Python packages

--------------------------

pip install tflite-runtime ultralytics

8. Verify Edge TPU delegate

---------------------------

python - << 'EOF'

from tflite_runtime import interpreter as tflite

print("Loading TPU...")

d = tflite.load_delegate('libedgetpu.so.1')

print("OK")

EOF

9. Fix NumPy and OpenCV versions

--------------------------------

pip install "numpy<2,>=1.23.2" --force-reinstall

pip install "opencv-python<4.10"

10. Place your models and scripts in ~/yolo

------------------------------------------

Example files:

- yolo11n_int8_edgetpu.tflite

- detect.py

- api.py

- logger.py

11. Run inference and API

-------------------------

python detect.py

python api.py